ESP32 Home Automation

MERN Stack + WebSocket + Next.js

# Overview

A full-stack home automation system built with Next.js (frontend), Node.js/Express (backend), MongoDB (database), and ESP32 microcontrollers. Devices are controlled in real time via WebSockets — no Blynk or third-party IoT platform required.

## Tech Stack

- Frontend: Next.js (React) with Socket.IO client

- Backend: Node.js + Express + Socket.IO + raw WebSocket server

- Database: MongoDB with Mongoose

- Hardware: ESP32 with ArduinoJson + WebSockets library

# Getting Started

## Prerequisites

- Node.js v18+

- MongoDB Atlas account (or local MongoDB)

- Arduino IDE with ESP32 board support

- Arduino Libraries: ArduinoJson (Benoit Blanchon), WebSockets (Markus Sattler)

## 1. Backend Setup

Navigate to the backend folder and install dependencies:

cd backend npm install

Create a .env file in the backend folder:

MONGO_URI=your_mongodb_connection_string JWT_SECRET=your_jwt_secret CLIENT_URL=http://localhost:3000 SERVER_URL=http://localhost:5000

Start the backend server:

npm run dev

## 2. Frontend Setup

Navigate to the frontend folder and install dependencies:

cd frontend npm install

Create a .env.local file in the frontend folder:

NEXT_PUBLIC_API_URL=http://localhost:5000 NEXT_PUBLIC_SOCKET_URL=http://localhost:5000

Start the frontend:

npm run dev

Open http://localhost:3000 in your browser.

## 3. ESP32 Setup

Open esp32_home.ino in Arduino IDE and update the config section at the top:

const char* WIFI_SSID = "YourWiFiName"; const char* WIFI_PASS = "YourWiFiPassword"; const char* SERVER_IP = "192.168.1.X"; // Your PC local IP (run ipconfig) const int SERVER_PORT = 5000; const char* DEVICE_ID = "esp32-home-01"; // Must match web app

Select Tools → Board → ESP32 Dev Module, then Tools → Port → COM4 (Silicon Labs CP210x). Upload the sketch.

# Pin Mapping

When adding pins in the web app, use numbers 1–4 (not D0, D1, etc.). Each number maps to a physical relay on the ESP32:

| Pin # in Web App | ESP32 GPIO | Relay | Button GPIO |
| --- | --- | --- | --- |
| 1 | GPIO 2 | Relay 1 | GPIO 26 |
| 2 | GPIO 4 | Relay 2 | GPIO 25 |
| 3 | GPIO 5 | Relay 3 | GPIO 33 |
| 4 | GPIO 18 | Relay 4 | GPIO 32 |

# System Architecture

The system uses two separate WebSocket connections on the same server:

Dashboard (React) ←─ Socket.IO ─→ Backend (Node.js) ←─ Raw WS ─→ ESP32 ws://localhost:5000/socket.io/ ¦ ws://localhost:5000/device

Control Flow

- Dashboard toggle → Socket.IO emit 'control' → Server → Raw WS → ESP32 fires relay

- Physical button press → ESP32 sends 'state_update' → Server → Socket.IO → Dashboard updates

- ESP32 connects → sends 'register' → Server adds to onlineDevices map → Dashboard shows Online

# Expanding to More Than 4 Pins

## Step 1 — Add GPIO definitions in .ino

#define RELAY5_PIN 19 #define RELAY6_PIN 21 #define RELAY7_PIN 22 #define RELAY8_PIN 23 const int relayPins[8] = { RELAY1_PIN, RELAY2_PIN, RELAY3_PIN, RELAY4_PIN, RELAY5_PIN, RELAY6_PIN, RELAY7_PIN, RELAY8_PIN }; // Change every loop from < 4 to < 8 for (int i = 0; i < 8; i++) { ... }

## Step 2 — Update device_page.jsx

// Change this line: const ALL_PINS = ['1', '2', '3', '4', '5', '6', '7', '8']

## Extended Pin Reference

| Pin # | GPIO (Relay) | Notes |
| --- | --- | --- |
| 1 | GPIO 2 | Original relay 1 |
| 2 | GPIO 4 | Original relay 2 |
| 3 | GPIO 5 | Original relay 3 |
| 4 | GPIO 18 | Original relay 4 |
| 5 | GPIO 19 | Extended — add to relayPins[] |
| 6 | GPIO 21 | Extended |
| 7 | GPIO 22 | Extended |
| 8 | GPIO 23 | Extended |

Note: Highlighted rows (pins 5–8) require hardware expansion. Make sure your relay module supports the additional channels.

## Alternative: Multiple ESP32 Devices

If you only have a 4-channel relay module, add a second ESP32 as a separate device in the web app with a different device ID:

const char* DEVICE_ID = "esp32-bedroom-02";

# Features

- Real-time relay control via WebSocket (no page refresh needed)

- Physical button support — pressing buttons updates the dashboard instantly

- Bluetooth fallback — control via Bluetooth Serial when WiFi is unavailable

- Device online/offline status shown in dashboard

- Voice command support via Web Speech API

- Multiple rooms and devices per account

- JWT authentication

# Troubleshooting

## Device shows Offline on dashboard

- Make sure .env.local uses localhost:5000, not a Railway/production URL

- Restart Next.js after changing .env.local (env vars are baked at build time)

- Check browser console for: [useSocket] 📋 online_devices: ['esp32-home-01']

## Toggle does nothing / relay does not fire

- Delete old D0/D1/D2 pins and re-add them with numbers 1, 2, 3, 4

- Check Arduino Serial Monitor — should show: Command received: pin=1 index=0 state=ON

- Verify SERVER_IP in .ino matches your PC's IPv4 address (run ipconfig)

## ESP32 keeps connecting and disconnecting

- Remove ws.enableHeartbeat() from setup() — conflicts with server ping interval

- Check Windows Firewall — run as Admin: netsh advfirewall firewall add rule name="Node 5000" dir=in action=allow protocol=TCP localport=5000

## Phone cannot reach server

- Run the firewall command above

- Use your PC's local IP in the phone browser: http://192.168.1.X:5000/health

- Both devices must be on the same WiFi network

# Health Check Endpoint

Visit this URL to verify the server is running and see connected devices:

http://localhost:5000/health

Expected response:

{ "status": "ok", "onlineDevices": ["esp32-home-01"] }

ESP32 Home Automation — University Project